PROJECT 2.4

The Electronic Gate — NPN Transistor

2N2222 NPN  |  LDR Voltage Divider  |  Base Resistor  |  Proteus ISIS Simulation

## 1. Project Overview

| Component | 2N2222 NPN Transistor |
| --- | --- |
| Purpose | Amplify small LDR signal to drive a larger load current |
| Key Concept | Amplification — Base controls Collector current |
| Input Signal | Voltage Divider output (LDR + R1 = 10 kΩ) |
| Base Resistor (Rb) | 10 kΩ — protects the Base pin |
| Collector Resistor (Rc) | 470 Ω — limits collector current |
| Load | LED (Vf ≈ 2.1 V) |
| Supply Voltage (VCC) | 5 V DC |
| Current Gain (hFE / β) | ≈ 100× |

## 2. Description

This project demonstrates how a tiny signal from the LDR (Light Dependent Resistor) — taken from the voltage divider built in Project 2.3 — can be used to trigger and control a much larger current flowing through the collector circuit. The 2N2222 NPN transistor acts as an electronic switch and amplifier.

## 3. Core Concept: Amplification

The transistor acts as a bridge (or gate):

- A small current at the Base (B) pin — controlled by the LDR voltage divider — opens the floodgates.

- This allows a much larger current to flow from Collector (C) to Emitter (E), lighting the LED.

- The ratio Ic / Ib is called hFE (current gain, β), approximately 100 for the 2N2222.

Key Formulas Vmid = Vcc x R1 / (LDR + R1) Ib = (Vmid - 0.7V) / Rb Ic = hFE x Ib (hFE ~ 100) Vled = 2.1V (forward voltage of LED)

## 4. Proteus Simulation Task

Using Proteus ISIS, assemble the following circuit:

- Place a 2N2222 NPN transistor on the schematic.

- Build the LDR + R1 (10 kΩ) voltage divider connected between VCC and GND.

- Feed the midpoint (Vmid) of the voltage divider through Rb = 10 kΩ into the Base pin.

- Connect an LED in series with Rc = 470 Ω between VCC and the Collector pin.

- Connect the Emitter pin directly to GND.

- Set VCC = 5 V and run the simulation. The LED should illuminate when the LDR detects light.

## 5. Circuit Diagram (Proteus ISIS Style)

Figure 1 — Project 2.4 Circuit: 2N2222 NPN Transistor Gate with LDR Voltage Divider

## 6. Component List

| Qty | Component | Value / Notes |
| --- | --- | --- |
| 1 | NPN Transistor | 2N2222 |
| 1 | LDR (Photoresistor) | ~5 kΩ in light, ~50 kΩ dark |
| 1 | Fixed Resistor (R1) | 10 kΩ |
| 1 | Base Resistor (Rb) | 10 kΩ (critical!) |
| 1 | Collector Resistor (Rc) | 470 Ω |
| 1 | LED | Any color, Vf ≈ 2.1 V |
| 1 | DC Power Supply | 5 V (VCC) |
| — | Wires / Breadboard | For connections |

## 7. Transistor Pin Functions (2N2222)

| Pin | Name | Function |
| --- | --- | --- |
| B | Base | Input control pin — receives Vmid via Rb. Vbe ≈ 0.7 V to turn on. |
| C | Collector | Main current path — connects to load (LED + Rc) and VCC. |
| E | Emitter | Output — connected to GND. Current exits here. |

## 8. Troubleshooting

⚠ WARNING: Missing Base Resistor Is the transistor getting hot? → You likely forgot the Base Resistor (Rb). Without Rb, the Base draws excessive current, causing thermal runaway and possible component destruction. Always use Rb = 10 kΩ to limit the trigger current!

| Problem | Likely Cause | Solution |
| --- | --- | --- |
| Transistor gets hot | No base resistor (Rb) | Add Rb = 10 kΩ at Base pin |
| LED does not light up | Vmid too low (dark room) | Increase light on LDR or check divider |
| LED always ON in dark | R1 too small vs LDR | Use larger R1 or reduce LDR value |
| Transistor stays OFF | Vbe never reaches 0.7 V | Check voltage divider output at Base |
| LED very dim | Rc too large | Reduce Rc or check LED polarity |

## 9. How It Works — Step by Step

- Step 1: Light hits the LDR: The LDR resistance drops (e.g. from 50 kΩ in dark to ~5 kΩ in light). This changes the voltage at the midpoint of the voltage divider (Vmid).

- Step 2: Vmid rises: As LDR resistance falls, Vmid = Vcc × R1/(LDR+R1) increases toward VCC.

- Step 3: Base current flows: When Vmid exceeds 0.7 V (Vbe threshold), current flows through Rb into the Base: Ib = (Vmid - 0.7) / Rb.

- Step 4: Transistor switches ON: The transistor enters the Active region. Collector current Ic = hFE × Ib ≈ 100 × Ib flows from VCC through Rc and the LED.

- Step 5: LED lights up: The large collector current drives the LED. The transistor has amplified the tiny LDR signal into a useful output.

## 10. Summary

Project 2.4 demonstrates the fundamental principle of transistor amplification. By feeding a light-sensitive voltage signal from an LDR voltage divider into the Base of a 2N2222 NPN transistor, we can control a much larger current in the Collector circuit — lighting an LED. The Base Resistor (Rb) is critical: it limits the Base current and prevents the transistor from overheating. This circuit is the foundation for countless real-world applications including light-activated alarms, automatic lighting systems, and sensor-controlled devices.